Printer Simulator User Manual

Prepared by Rahul Gupta

Contact email – rgupta22@binghamton.edu

Contents

List of Figures ……………………….…………….………  3

List of Tables ……………………….…………….………..  3

- About the dataset ……………………………...  4 - 5

- Printer simulator process flow ………............   6

- Software Requirements…………………….….  7

- Steps to use printer simulator.………….…....... 8

- Result in a file…………….……………….……  9

- Guidelines about the code ...………….………  10 - 11

List of Figures

- PCB board MOM4 ………………………………….……………… 4

- PCB board EKRA ..……………………………….………………… 5

- Flowchart of printer simulator …………………….……………… 6

- Flowchart of generalizing simulator using transfer learning ….  7

- Input parameters ……………………………………….................... 9

- Input full-scale SPI data ...………………………………................. 9

- Input new job file SPI data ...………………………………............. 9

- The output of simulation ….………………….….……................... 9

- The Output of simulation with transfer learning ………………. 10

- Output Excel sheet …….……………………………….................... 11

- Output with Transfer Learning Excel sheet ……………………... 11

- Output file name …….…………………………………................... 11

- Input feature for which you want simulation ………………….. 12

- Decomposition and reconstruction code for noise …………….. 12

- Random noise generation code ...........................………………... 12

- Prediction code.……………………………………………………. 13

- Transfer learning code for new job file ..……………………… 14

List of Tables

- MOM4 pads dimension ……………………...…………………… 5

- About the dataset

  - Collected from SPI consist of:

    - PadID, Size X, Size Y, Volume (%), OffsetX(um), OffsetY(um), PCB ID, Printing Speed, Printing Pressure, Separation Speed, Cleaning Type, Cleaning Age, Direction, Pos X, Pos Y, Rotation, AR, ASR

    - AR- Area ratio

AR=Area of padArea of aperture walls

    - ASR- Aspect ratio

AR=Aperture widthStencil thickness

  - 125 printer settings (5 printing pressure, 5 printing speed, 5 separation speed)

    - Printing pressure – 60, 65, 70, 75, 80

    - Printing speed – 25, 30, 35, 40, 45

    - Separation speed – 3, 3.5, 4, 5.5, 7

  - 5 boards printed for each setting, since used cleaning cycle length of 5 boards

  - Clean stencil before each setting changing with sequence (Wet-Dry-Dry)

  - The board used for simulation is MOM4

Figure1. PCB board MOM4

    - MOM4 board has pad dimensions.

|  | 0402 |  | 0603 |  | 1005 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | x(mm) | y(mm) | x(mm) | y(mm) | x(mm) | y(mm) |  |
| Pad dimension | 0.22 | 0.22 | 0.33 | 0.33 | 0.56 | 0.56 |  |

Table1. MOM4 pads dimension

  - Dataset collected from SPI for Transfer learning to adapt for different board type and printer type consist of:

    - PadID, Size X, Size Y, Volume (%), PCB ID, Printing Speed, Printing Pressure, Separation Speed, Cleaning Age, Direction, Pos X, Pos Y, AR, ASR

    - 8 printer settings (PCB) (4 printing pressure, 2 printing speed, 4 separation speed)

      - Printing pressure – 60, 65, 70, 75

      - Printing speed – 25, 40

      - Separation speed – 3.5, 4, 4.5, 5.5

    - The board used for validation of simulation when used with transfer learning is EKRA printed with EKRA printer.

Figure2. PCB board EKRA

- Printer simulator process flow

Figure3. Flowchart of printer simulator

Figure4. Flowchart of generalizing simulator using transfer learning

- Software Requirements

  - Software

    - Python (Version 3.9 or higher)

    - Four libraries are required

        - pip install pywavelets

        - pip install numpy

        - pip install pandas

        - pip install scikit-learn

        - pip install tensorflow

- Steps to use printer simulator

  - Open python file

  - Input

Figure5. Input parameters

- Cleaning type= [Wet, Dry]

- Direction= [F, B]

  - Input SPI data files with 624 boards information

Figure6. Input full-scale SPI data

  - Input new training data files for retraining transfer learning model with 8 boards information

Figure7. Input new job file SPI data

  - Run the printer simulator code

  - Output will be generated

Figure8. The Output of simulation

Figure9. The Output of simulation with transfer learning

- Result in a file

  - Results are stored in an Excel file (Simulator Output.xlsx) as below. And it’s downloaded to the local system.

Figure10. Output Excel sheet

Figure11. Output with Transfer Learning Excel sheet

  - We can change this file name from the code

Figure12. Output file name

- Guidelines about the code

  - Use Input in 3D (PosX×PosY×No_Of_PCB)

    - As the level of decomposition is 2, we should use even number of PCB, as in our case, it’s better to use 624

Figure13. Input feature for which you want simulation

  - DTCWT is used to get noise from the real printer. Below is the transformation of data into noise and true signal

Figure14. Decomposition and reconstruction code for noise

  - Random noise was generated for each similar pad group and aggregation with true volume to get simulated volume

Figure15. Random noise generation code

  - If board type and printer used is same as input data used for simulator

    - Decision tree used for prediction for input parameters which generalize simulator

    - X – Input feature (Printing Speed, Printing Pressure, Separation Speed, Cleaning Age, AR, ASR)

    - Y – Output feature (Volume (%) / OffsetX(um) / OffsetY(um))

Figure16. Prediction code

  - If board type and printer type used is different from input data used for simulator: used transfer learning with neural networks

    - Pre-training: The model is initially trained (pre-trained) on a simulated dataset to learn general patterns

    - Fine-tuning: Subsequently, the pre-trained model is fine-tuned on a smaller, new job file dataset. This approach leverages the knowledge gained during pre-training to improve prediction accuracy on the new job file dataset

Figure17. Transfer Learning Code for new Job File

  - Noise level is controlled by the threshold of DTCWT, if required more variation increase the threshold and vice versa.

  - To know more about the Dual-Tree Complex Wavelet Transform (DTCWT) use the reference given below

    - I. W. Selesnick, R. G. Baraniuk and N. C. Kingsbury, "The dual-tree complex wavelet transform," in IEEE Signal Processing Magazine, vol. 22, no. 6, pp. 123-151, Nov. 2005, doi: 10.1109/MSP.2005.1550194.